# Matlab scripts Laboratory of Fluid Mechanics

18 July 2005

In the Laboratory of Fluid Mechanics, some Matlab scripts to be used in combination with the experiments have been written. The scripts are listed below (the folder names between the parentheses are subfolders on the CD containing Matlab scripts):

- Data processing scripts (folder Lab_process).

Scripts to evaluate and process the measurements, especially data from wave gauges.

Some of these scripts can be called from a user interface named Labmenu.

- Extra dat processing scripts, added afterwards (subfolder Lab_prcess\Extra_s)

- Scripts to generate test signals (subfolder Lab_process\testsig).

Scripts to generate test input to test the reflection programs.

- Programs to be used by employers of the Lab (folder Lab_hulp).

Scripts to perform specific tasks, especially intended to be used by employers of the Laboratory of Fluid Mechanics in experiments and test equipments. One of those scripts is making graphs of the discharge over a weir.

- Administration scripts (folder Admin).

These scripts are intended to make lists of script names and function names, a contents.m-

file, and a list of function calls of each .m-file.

Usage: type Admin, and a menu appears (description at the end of this document).

## Usage of the scripts

The scripts can be used from Matlab by going to the script's folder (not the folder on the cd!), and typing the script name. Some scripts from the folder \Lab_process can be started from a user interface, by going to the folder where the contents of \Lab_process has been stored, and typing Labmenu. From this user interface, the parts View, Tools, and Help can be chosen. Table 1 shows which script can be started from the parts of this user interface. All scripts can also run independently, without the user interface. Information about the use of the scripts from the user interface: type Labmenu, and click on Help, or look in \Lab_process\Help\Usage6.doc.

Note:	Never run scripts directly from the cd. In that case, the program will try to write data to the cd, and an error message will appear.

Table 1: scripts to be started from Labmenu

| Labmenu part | Script to be started | Purpose |
| --- | --- | --- |
| View, measurements (*.asc) | Lvmview | Menu to view measure-ments from ASCII-file |
| View, steerfile (*.dat) | Bekijkb | Visualise a steerfile, generated by Multilin (DELFT-AUKE) |
| Tools, Convert Ascii to Auke | Asc2wav | Graphical user interface around script Asc2wav |
| Tools, Refl. ir. wave | Decomp | Separate irregular wave (continuous spectrum) in incoming and reflected wave |
| Tools, Refl. reg. wave | Refreg | Separate regular wave in incoming and reflected wave |
| Tools, variance spectrum | Pwspect | Calculate variance spectrum for each wave gauge |
| Tools, amplitude spectrum | Spect | Calculate amplitude spectrum for specified channel (without phase); to calculate energy per f-band, use Tools, variance spect |

In most of the scripts, parameters are asked from the screen. In some cases, the parameters are read from a special input file.

The data to be processed must be stored in an ASCII-file. The next conditions must be fulfilled:

- The header may have a free number of lines. The number of header lines will be asked from the screen. In case of a Dasylab file (from the data acquisition program used in the Lab.), the length of the header will be determined "automatically".

- All lines must contain an equal number of channels.

- The channels can be separated by a tab, a space, a comma, or a semicolon.

The next errors may occur:

- If the number of header lines specified in case of a non-Dasylab file is less than the actual number, the read function used in the scripts gives a message. However, in some cases Matlab gives the next error:

Number of colums in line -- of ASCII file ... must be the same as the previous line.

In such a case, remove all spaces, signs, or numbers from the first position of each header line.

If the number of header lines specified is more than the actual number, no message will occur, but less data lines will be read.

- If a Dasylab file gives an error like above, check if the right file has been chosen. If yes, copy this file, delete the first line, and read the copy specifying the remaining number of header lines.

- The maximum number of samples to be read is exceeded. This number is set to 500000/nchan,

where nchan is the number of channels used. If more data are needed, change the value of maxnum in \Functions\FN_indata7, line 86, or read the file in parts, using different sample numbers. Sample numbers to be read can be specified via the dialog box:

read all samples?

Specify No in this case.

In a Dasylab file, it is preferable to store the time in decimal format. The time format with a ':' in Dasylab is not read as a time in Matlab.

For some scripts, a short manual has been written. From the Labmenu, Help part, information and manuals can be viewed as well.

In the remainder part of this manual, direct usage of all scripts will be explained. Most of the description can be used if the scripts are started from the user interface as well.

The next pages show tables of purpose, input and output, and file name of the manual if available. The tables are ordered per script type. The manuals are stored on the CD, in a subfolder Docs of each folder on the CD.

Table 2.  Data processing scripts (bold scripts are to be used from user interface Labmenu as well).

| Script name | Purpose | Input | Output | Manual |
| --- | --- | --- | --- | --- |
| Asc2wav | Convert ASCII file to binary AUKE file, and generate command file for DELFT-AUKE program Waves | Data file (*.asc); Parameters from screen | AUKE file (*.dat, *.seq) | Man.doc |
| Bekijkb | Visualise a steerfile, generated by Multilin (DELFT-AUKE) | Steerfile (.dat); Parameters from screen | Graph of data specified by user (from dialog box) |  |
| Decomp | Separate irregular wave (continuous spectrum) in incoming and reflected wave | .txt-file with parameters about the data file; .txt-file with depth profile; Data file (.asc) | Spectra and/or time-series of incoming and reflected wave | Man.doc |
| Jonswap1 | calculate freq-range of an ideal Jonswap spectrum | usage: Jonswap1 parameters from screen | graph of spectrum, print f-range on screen |  |
| Lvmsp | Visualise data from a data file using sptool from the signal processing Toolbox | Data file (*.asc); Parameters from screen | Sptool will be opened | Book: Matlab signal processing Toolbox |
| Lvmview | Menu to view measure-ments from ASCII-file | Data file (*.asc); Parameters from screen | Graph of desired samples from desired channels |  |
| Process | Frame work of a script to read and process a data file | Data file (*.asc) | Graph of channels; User has to add his own output files |  |
| Pwspect | Calculate variance spec-trum for each wave gauge | Data file (*.asc); Parameters from screen | Graph of variance spectrum per gauge; .txt-file containing wave parameters; .asc-file containing spectra | Usage is same as Decomp |
| Refreg | Separate regular wave in incoming and reflected wave | Data file (*.asc); Parameters from screen | .txt-file containing wave parameters; | Man.doc |
| Refreg1 | Same as Refreg, input from file Inparms.m | File containing input parameters (Inparm.m); Data file (*.asc) | .txt-file containing wave parameters; |  |
| Spect | Amplitude spectrum of 1 chanel | Data file (*.asc); Parameters from screen | .txt-file containing amplitude spectrum; plot of ampl.spectrum |  |
| Wavper, Wavper2 | Compute wavelength and wave period as a function of waterdepth, wavelegth, and wave steepness | Wavper: via screen, Wavper2: direct in script | Wavper1: via screen, Wavper2: .txt-file with tables |  |
| Wavel | Compute wavelength for given water depth and wave period from dispersion relation | Put directly into the script (h and T as vector) | Table of h, T, and corresponding wavelength |  |

Each script in Table 2. has been tested using a specific input file.

The input files have been stored in .\Indata, and the output from the tests has been stored in .\Outdata.

List of input and output files per script:

| script name | input files | output files |
| --- | --- | --- |
| Asc2wav | Jsh10.asc | Jsh10.dat Jsh10.seq Waves.pcf |
| Bekijkb | Reg.dat Irreg.dat |  |
| Decomp | 2 wave gauges |  |
|  | Jsh10.asc (data) Jsh10.txt (parameters) Jsh10prof.txt (depth profile) | Jsh10.txt (parameters) Jsh10sp.asc (variance spectra) Jsh10dc.asc (inc. and refl. wave as a time signal) |
|  | 3 wave gauges |  |
|  | H09s4.asc (data) H09s4.txt (parameters) H09s4prof.txt (diepteprofiel) | H09s4.txt (parameters) H09s4sp.asc (variance spectra) H09s4dc.asc (inc. and refl. wave as a time signal) |
| Lvmsp | Jsh10.asc |  |
| Lvmview | Jsh10.asc |  |
| Process | Jsh10.asc |  |
| Pwspect | Jsh10.asc | Jsh10pwsp.asc |
| Refreg | Refreg.asc test.asc | Refreg.txt |
| Regreg1 | Refreg.asc | uitRefreg.asc (chosen automatically; stored in input folder) |
| Spect | Refreg.asc | Spect.txt |
| Wavel | Input directly into Wavel.m |  |

Information about the files in  .\Indata\Contents.txt

Table 3.  Scripts to generate test signals

| script name | purpose | input | output | manual |
| --- | --- | --- | --- | --- |
| testsig3 | simulation of regular waves, 2 wave gauges, more output files per run possible | text file (.txt) containing wave parameters | Header in name.txt, data in name.asc (name: input parameter), All output will be stored in the input folder |  |
| testsig4 | Simulation of regular waves, 2 wave gauges, one output file | Asked from screen | test.txt, test.asc in working folder (here: \Lab_process\testsig) |  |
| testsig7 | Simulation of irregular wave, free number of wave gauges, one output file | Text file (.txt) containing wave parameters, ASCII file containing irregular wave, generated by DelftAUKE (Wave Wanted, ASCII) | ASCII file in folder chosen by dialog box. |  |

All output of the examples stored in \Indata on the CD can be found in \Outdata.

Tabel 4.  Scripts added later

| scriptname | purpose | input | output | manual |
| --- | --- | --- | --- | --- |
| map \Lab_process\Extra_s\Pos2vel |  |  |  |  |
| Vels | calculate velocities and accelerations from positions | output file from Multiscan_xy, A.M. den Toom | header in name.txt, results in name.asc (name: asked via screen) | subfolder \Docs |
| Vels5 | older version of Vels, calculation in different order (see \Docs) | see Vels | see Vels |  |
| folder \Lab_process\Extra_s\Refreg_ext |  |  |  |  |
| Refreg19 | Separate regular wave in incoming and reflected wave, after substraction of moving average over length of base period | data file (*.asc); parameters via screen | .txt-file with wave parameters | Man.doc |

All output of the examples stored in \Indata on the CD can be found in \Outdata.

Table 5.  Programs to be used by employers of the lab.

| Script name | Purpose | Input | Output | Manual |
| --- | --- | --- | --- | --- |
| Biesel | Graph and table of the Biesel function as a function of the frequency for different water depths | Put directly into the script (f and h as vector) | .txt file containing tables; Graph of the Biesel function |  |
| Biesel3 | Graphs of the maximum wave height and the Biesel function as a function of the frequency for different water depths | Put directly into the script (f and h as vector) | Graphs |  |
| Dat2asc | Convert Multilin steerfile to Ascii file with Dasylab header | Data file (*.asc); Parameters from screen | Ascii file (*.asc) with Dasylab header; steervalues are stored only [-10V,10V] |  |
| Debiet | Calculation of the discharge of a weir as a function of the height h above the top | Asked from the screen | Graph; RTF file containing tables | Man.doc (description of Debiet in Dutch) |
| Hmax | Graph of the maximum wave height as a function of the frequency for different water depths | Put directly into the script (f and h as vector) | Graphs |  |
| Meetfl | Table of discharge of a flange | Put directly into inputfunction | table | Meetfl.txt (in Dutch) |
| Wavechars | Wavelength and wave steepness for given water depth, wave period, and wave height | Put directly into the script (h, T and H as a vector) | Table in .txt file |  |

Administration scripts

The main script in this folder is Admin.m. After typing "Admin", a screen will be shown with "File" in the upper left corner. Click on File, Open; a pull down menu will be shown, having parts:

Proglist	Makes a list of files of a specified extension from a specified folder; extension used in most cases: .m to produce a contents.m file or a list of function calls.

Contents	Makes a contents.m file. Before starting Contents, the user must make a list of .m-files  using Proglist (previous option). From all .m-files, the first 8 lines will be put into Contents.m. This is only meaningful if the first 8 lines contain a description of the script or function.

Calls	Makes a list of function calls of each script or function. Before starting Calls, the user must make a list of calling scripts and functions and a list of called functions (using Proglist). The second list may be the same as the first. In this way, all functions of the second list called by each script or function of the first list can be found. This option has been added to find out which older functions can be deleted.

View	Shows a text file on the screen; by this option the result of one of the foregoing options can be viewed.