Matlab tools Laboratory of Fluid Mechanics

Usage of Labmenu	2

Pulldown menus on the toolbar of Labmenu	2

Usage of the View and Tools pulldown menus	2

Conditions of data files (ASCII) to be read	3

Purpose of the menu items	4

Input, and output of the menu items	5

Examples	6

View: Measurements (Ascii)	6

View: Steerfile (*.dat)	7

Tools: Convert Ascii to Auke	8

Tools: Refl. ir. wave	9

Tools: Refl. reg. wave	10

Tools: Variance spectrum	11

Tools: Amplitude spectrum	12

# Usage of Labmenu

15 June 2005

Back to start

## Pulldown menus on the toolbar of Labmenu

N.B.:	This program uses dialog boxes. In Matlab 7, dialog boxes will not be deleted immediately after clicking OK, but the program will continue; please don’t click OK again.

- View: view a data file (ASCII) or a steerfile from DELFT AUKE (*.dat)

- Tools: open a script to process the data file

- Help: show help file

## Usage of the View and Tools pulldown menus

Back to start

In most of the menu items, parameters are asked from the screen. In some cases, the parameters are read from a special input file.

The data to be processed must be stored in an ASCII-file. The conditions are listed in Conditions of data files (ASCII) to be read.

The options: View: measurements (ASCII) and Process: Refl. reg. wave will open submenus. From those menus data will be read into the workspace, and parameters can be changes by choosing options.

N.B.	After closing a (sub)menu, allways check the number of Matlab Windows (see Matlab icon in lower bar of monitor), and close all windows left open (except Labmenu).

Don’t forget to jump back to a dialog box if help is consulted, and the box is left open.

## Conditions of data files (ASCII) to be read

Back to start

- The header may have a free number of lines. The number of header lines will be asked from the screen. In case of a Dasylab file (from the data acquisition program used in the Lab.), the length of the header will be determined "automatically".

- All lines must contain an equal number of channels.

- The channels can be separated by a tab, a space, a comma, or a semicolon.

The next errors may occur:

- If the number of header lines specified in case of a non-Dasylab file is less than the actual number, the read function used in the scripts gives a message. However, in some cases Matlab gives the next error:

Number of colums in line -- of ASCII file ... must be the same as the previous line.

In such a case, remove all spaces, signs, or numbers from the first position of each header line.

If the number of header lines specified is more than the actual number, no message will occur, but less data lines will be read.

- If a Dasylab file gives an error like above, check if the right file has been chosen. If yes, copy this file, delete the first line, and read the copy specifying the remaining number of header lines.

- The maximum number of samples to be read is exceeded. This number is set to 500000/nchan,

where nchan is the number of channels used. If more data are needed, change the value of maxnum in \Functions\FN_indata7, line 86, or read the file in parts, using different sample numbers. Sample numbers to be read can be specified via the dialog box:

read all samples?

Specify No in this case.

In a Dasylab file, it is preferable to store the time in decimal format. The time format with a ':' in Dasylab is not read as a time in Matlab.

For some scripts, a short manual has been written. The manuals can be viewed via Help, Manuals in Labmenu.

Back to start

## Purpose of the menu items

Back to start

| Menu Item | Purpose | Manual |
| --- | --- | --- |
| View: Measurements | Menu to view measurements from ASCII-file |  |
| (ASCII) |  |  |
| View: Steerfile | Visualise a steerfile, generated by Multilin |  |
| (*.dat) | (DELFT-AUKE) |  |
| Tools: | Convert ASCII file to binary AUKE file, and generate | Man.doc |
| Convert ASCII to Auke | command file for DELFT-AUKE program Waves |  |
| Tools: | Separate irregular wave (continuous spectrum) in | Man.doc |
| Refl. ir. wave | incoming and reflected wave |  |
| Tools: | Separate regular wave in incoming and reflected | Refreg_eng.doc |
| Refl. reg. wave | wave |  |
| Tools: | Calculate variance spectrum for each wave gauge | Usage is same as |
| Variance spectrum |  | Decomp |
| Tools: | Calculate amplitude spectrum of a specified channel (1 channel); |  |
| Amplitude spectrum | Use in case of reg. wave with more harmonics; for energies, use Variance spect. |  |

Back to start

## Input, and output of the menu items

Back to start

| Menu Item | Input | Output |
| --- | --- | --- |
| View: Measurements | Data file (*.asc); | Graph of desired samples from |
| (Ascii) | Parameters from screen | desired channels |
| View: Steerfile (*.dat) | Steerfile (.dat); | Graph of data specified by user |
|  | Parameters from screen | (from dialog box) |
| Tools: | Data file (*.asc); | AUKE file (*.dat, *.seq) |
| Convert Ascii to Auke | Parameters from screen |  |
| Tools: Refl. ir. wave | .txt-file with parameters about the | Spectra and/or time-series of |
|  | data file; | incoming and reflected wave |
|  | .txt-file with depth profile; Data file (.asc) |  |
| Tools: Refl. reg. wave | Data file (*.asc); | .txt-file containing wave |
|  | Parameters from screen | parameters; |
| Tools: | Data file (*.asc); | Graph of variance spectrum per |
| Variance spectrum | Parameters from screen | gauge; |
|  |  | .txt-file containing wave parameters; .asc-file containing spectra |
| Tools: | Data file (*.asc); | .txt-file of ampl. spectrum; |
| Amplitude spectrum | Parameters from screen | graph of amp. spectrum (no phase) |

Back to start

## Examples

Back to start

Each menu item has been tested using a specific input file.

The input files have been stored in .\Indata, and the output from the tests has been stored in .\Outdata.

Examples using specific input and out for each menu item:

### View: Measurements (Ascii)

Back to start

N.B.:	This program uses dialog boxes. In Matlab 7, dialog boxes will not be deleted immediately after clicking OK, but the program will continue; please don’t click OK again.

N.B.	After closing a (sub)menu, allways check the number of Matlab Windows (see Matlab icon in lower bar of monitor), and close all windows left open (except Labmenu).

Don’t forget to jump back to a dialog box if help is consulted, and the box is left open.

- Click on File,  Open

- Search for subfolder Input, and click on Jsh10.asc

- Click on Open

- Answer the next questions:

| Read all samples? | Yes (use No in case of files of 20MB or more only) |
| --- | --- |
| Number of header lines | 0 (because this is a Dasylab file) |
| time step in org. data file | 0.1 |
| column with times | 1 (will be used to label the X-axes) |

- Click on OK; a select box will appear; select the desired channels by clicking on the channels with the left mouse button and pressing <Ctrl>

- A screen with all samples read will be displayed; from this, chose the time range of the samples to be viewed

- The samples to be viewed are from 150 s to 165 s for Jsh10.asc

- Click on OK, and close the figure of the scans to be analysed

- A message box “click Plot on menu bar to make your choise” will appear

- Close the message box; click Plot on the menu bar; to view the plot choose one of the options “Plot channels separated, 3 per fig” or “Plot channels in 1 fig”; the other options can be used to change the samples or channels chosen before

- One or more figures will be opened; if you want to change the samples viewed,

close the figures, and click on Plot: the samples or the figure type can be changed here

- After viewing the data, a new file can be opened

- The program can be closed be File, Close or by clicking the cross in the upper right of the screen;

you will be back in the main Labmenu.

Back to start

### View: Steerfile (*.dat)

Back to start

N.B.:	This program uses dialog boxes. In Matlab 7, dialog boxes will not be deleted immediately after clicking OK, but the program will continue; please don’t click OK again.

- In the box Input file, search for subfolder Input, file Irreg.dat

- Click on Open

- Answer the next questions:

| maximum number of samples | 100 (-1 to end of file) |
| --- | --- |
| type of wave board (h hydraulic, e electrical) | h |
| units (val value, n normed value, V Voltage, m amplitude of board in m) | V |

- A screen with all samples read will be displayed; from this, chose the time range of the samples to be viewed

- The samples to be viewed are from 60 s to 70 s for Irreg.dat

- Click on OK

- The range chosen will be displayed; after closing the figure, the output figure form “View Steerfile” will appear

- After closing the figure, you will be back in the main Labmenu

- Changes can be made only by start the option View: steerfile again.

Back to start

### Tools: Convert Ascii to Auke

Back to start

N.B.:	This program uses dialog boxes. In Matlab 7, dialog boxes will not be deleted immediately after clicking OK, but the program will continue; please don’t click OK again.

- In the box Input file, search for subfolder Input, and click on Jsh10.asc

- Click on Open

- Answer the next questions:

| Read all samples? | Yes (use No in case of files of 20MB or more only) |
| --- | --- |
| number of header lines | 0 (because this is a Dasylab file) |
| time step in org. data file | 0.1 |
| water depth | 0.6 |
| number of columns to be analysed | 4 |

- Next, the channel values will be asked in successive screens:

column number:	2	(3, 4, and 5 in next screens)

x-cd:			8	(8.25, 15, 15.25 in next screens)

scale factor:		0.025	(in all screens)

- A screen with all samples read will be displayed; from this, chose the time range of the samples to be analysed

- The samples to be analysd are from 150 s to -1 (end of file) for Jsh10.asc

- the name of the output file (*.seq) will be asked; type the full name, ore chose an existing .seq file

(Note: the Dutch Windows version of Windows may hang if you try to overwrite an existing file)

- The program will generate a .dat, a .seq, and a waves.pcf file.

- More information in Help, Manuals, Convert Ascii to Auke, or in file \Docs\Wave_eng.doc.

Back to start

### Tools: Refl. ir. wave

Back to start

N.B.:	This program uses dialog boxes. In Matlab 7, dialog boxes will not be deleted immediately after clicking OK, but the program will continue; please don’t click OK again.

Two input files are available: Jsh10.asc with 2 wave gauges, and H09s4.asc with 3 wave gauges.

Here only the file name and samples to be analysed will be asked from the screen. All othe parameters will be asked from .txt-files. Copy the text files, and supply your own case if you want to use your own measurements. More information in Help, Manuals, Refl. ir. wave, or in file \Docs\Decomp_eng.doc.

- After clicking “Refl. ir. wave”, a screen containing output options will be opened; put 1 in the box of the output options you want to see (and 0 in the box of the options you don't' want)

- In the box Data file, search for folde Input, and chose Jsh10.asc or H09s4.asc

- Click on Open

- Answer the next questions:

Read all samples?		Yes (use No in case of files of 20MB or more only)

number of header lines:		0 (because this is a Dasylab file)

- Click on OK

- Wait until the next screen appears (this will take a few seconds)

- The File containing parameters of data file is jsh10a.txt or H09s4.txt, depending on your first choise

- The File containing profile of depth is jsh10prof.txt or H09s4prof.txt, depending on your first choise

- A screen with all samples read will be displayed; from this, chose the time region to be analysed

- The time region to be analysed is 150 to -1 for Jsh10.asc, and 1000 to 10000 for H09s4.asc

- Click on OK; close the figure of the scans to be analysed to start the calculation

- During the calculation, some waitbars inform the user about the progress; wait till the program asks for output or till the last figure is displayed

- The output chosen in the first step of Refl. ir. wave will be produced; the name(s) of the output file(s) will be asked by screens; close all files and figures before going to the next program.

(Note: the Dutch Windows version of Windows may hang if you try to overwrite an existing file)

More information: Help, Manuals, Refl. ir. wave, or in file \Docs\Decomp_eng.doc.

Back to start

### Tools: Refl. reg. wave

Back to start

N.B.:	This program uses dialog boxes. In Matlab 7, dialog boxes will not be deleted immediately after clicking OK, but the program will continue; please don’t click OK again.

N.B.	After closing a (sub)menu, allways check the number of Matlab Windows (see Matlab icon in lower bar of monitor), and close all windows left open (except Labmenu).

Don’t forget to jump back to a dialog box if help is consulted, and the box is left open.

- Click on File,  Open, Data file

- Search for subfolder Input, and click on Refreg.asc

- Click on Open

- Answer the next questions:

| Read all samples? | Yes (use No in case of files of 20MB or more only) |
| --- | --- |
| number of header lines | 0 (because this is a Dasylab file) |
| timestep in org. data file | 0.02 |
| number of harmonics to be printed | 5 |
| average waterdepth | 0.5 |
|  | OK |
| column number | 6 |
| x-cd in m | 0 |
| scale factor measurement values to m | 0.0243 |
| column number | 5 |
| x-cd in m | 0.6 |
| scale factor measurement values to m | 0.0243 |

- Click on OK

- A screen with all samples read will be displayed; from this, chose the time region to be analysed

- The time region to be analysed is 10 to 90 for Refreg.asc

- Click on OK, and close the figure of the scans to be analysed

- The calculation will start, and the program will ask where to put the output file

- If the data to be analysed are not correct click on Cancel, and next on Calculate in the Toolbar of Refreg18 (the program calculationg refl. reg. wave); the parameters can be changed. You can use File to choose another file

- The output file will be displayed on the screen; close the file after viewing the results; the output file can be viewed afterwards by File, Open, Results file in the Toolbar of Refreg18

- Close the program by clickin File, Close, or by clicking the cross in the upper right of the screen;

you will be back in the main Labmenu.

Back to start

### Tools: Variance spectrum

Back to start

N.B.:	This program uses dialog boxes. In Matlab 7, dialog boxes will not be deleted immediately after clicking OK, but the program will continue; please don’t click OK again.

- In the box Data file, search for folde Input, and chose Jsh10.asc

- Click on Open

- Answer the next questions:

| Read all samples? | Yes (use No in case of files of 20MB or more only) |
| --- | --- |
| number of header lines | 0 (because this is a Dasylab file) |
| time step in org. data file | 0.1 |
| desired frequency step | 0.02 |
| max. frequency in graph | 2 |
| column with times | 1 |
| number of columns to be analysed | 2 |

- Next, the channels values will be asked in successive screens:

column number:	2	(3 in next screen)

scale factor:		0.025	(in all screens)

- Select time series to be analysed:

type the time in seconds of the first and last sample to be used in the calculation (here: 150 and -1 resp.)

- Close the figure; the spectra will be displayed; the program will ask for output file (spectral parameters and spectral values resp.).

Back to start

### Tools: Amplitude spectrum

Back to start

N.B.:	This program uses dialog boxes. In Matlab 7, dialog boxes will not be deleted immediately after clicking OK, but the program will continue; please don’t click OK again.

This option is of interest in case of a regular wave, because higher harmonics can be seen; in all other cases, the option Variance spectrum is preferrable, because amplitudes do not represent the amount of energy per frequency band; the phase must be included if using amplitudes.

- In the box Data file, search for folder Input, and chose Refreg.asc

- Click on Open

- Answer the next questions:

| Read all samples? | Yes (use No in case of files of 20MB or more only) |
| --- | --- |
| number of header lines | 0 (because this is a Dasylab file) |
| time step in org. data file | 0.02 |
| threshold in spectrum; plot part of amp>thresh*amp_max | 0.005 |
| number of channels to be analysed | 2 |
| column number | 5 (6 for second channel) |
| scale factor | 0.0243 |
| units | m |

- Click on OK

- Select time series to be analysed:

type the time in seconds of the first and last sample to be used in the calculation (here: 150 and -1 resp.)

- Close the figure to start the calculation; change of the samples selected above can only be done in a next run of the program; the calculation cannot be stopped here

- The location and name of the output file are asked; type a new name, or click on Cancel to skip this part of the program

- The amplitude spectrum will be displayed on the screen.

Back to start