# ESEM DE, comments and pictures

ESEM pictures Diatomaceous Earth (Elche), 14-04-2015, TU Delft

Comments on all pictures, sorted by layer/grade.

First the four grades (1A, 1B, 2A, 2B, 3A, 3B), than the 11 DE samples and lastly four IBP samples.

Some shapes of diatoms are different. These could be different types of diatoms, but I think the shape (amount and size of pores, outer rings etc.) were also changed due to processes after deposition. The sharp boundary between calcite and diatomite is interesting.

I will try to do some statistical work on these pictures with image analysis software. And compare these results with the data from LIBS and NIR analysis.

Crude Ore 1A

CO 1A doesn’t have any white dots on the diatoms. The amount of broken fragments is medium. The picture with 500x and the left 1000x enlargement show something interesting. It is either a cross section of a diatom or a different type. The material looks as if it is made of almost only diatoms. A longer thing (pennate?) can be observed in the first picture. The diatoms seem to be very clean.

Crude Ore 1B

Sample Crude Ore 1B is less well sorted than sample 1A. One can see needles of probably calcite. In some places it is very ‘open’. There are many diatoms without an outer ring, but there are not many loose rings. There are not many broken fragments. Some diatoms have white dots in a circle on the diatom, but mostly they don’t have anything. The diatoms are not as clean as in CO 1A.

Crude Ore 2A

Sample CO 2A has no white dots on the diatoms. There are many broken fragments of both the inner and outer circles. It also looks as if there is also other material present apart from diatoms. There are also many parts of diatoms with only a little amount of pores. It looks as if pores are filled after deposition.

Maybe there are primary and secondary pores?

Crude Ore 2B

Sample CO 2B is very densely packed. There are not many white dots. The amount of broken fragments is medium. A pennate can be observed, not only from the side but also the cross section. There are not many pores in the diatoms and they have a large cross section. There are large and small diatoms, the material does not look homogeneous.

Crude Ore 3A

There are no white dots in sample CO 3A. There are many broken fragments. One can observe needles of crystals that are probably calcite. The pores are small, the calcite needles look to be mostly in between the diatoms and not in the structure of the diatoms itself. The orientation of the diatoms is very random.

Crude Ore 3B

Sample CO 3B is very homogeneous and consists almost completely of diatoms. The diatoms are mostly orientated in the same direction. The diatoms have many white dots in a circle on the outside of the inner ring of the diatom. In the 500x picture one can see a cross section or different type of diatom in the centre of the picture. Also stacks of diatoms can be observed in the 1000x picture. Almost all diatoms are intact and there are no broken fragments.

DE 1

Sample DE1 shows some interesting features. The first two pictures show the boundary between a layer of almost pure diatomite and a layer of almost all pure other material, probably calcite. The boundary is extremely sharp and very straight. This boundary can also be observed in the thin sections and by the naked eye.

The material consists of a mix of intact diatoms and broken fragments. There are no white dots present. The diatomite layer looks well sorted and most of the diatoms are intact.

The needles of calcite look like they had time and space to grow.

The sharp boundary could indicate a sudden event in which the diatoms died and the calcite started to grow. This event could be a change in environment, for example salinity or temperature.

DE 2

In sample DE 2 there again is a boundary between the two types of material.  There is more broken material than in sample DE 1. There are pennates present, for example in picture 2. There is much calcite in between the diatoms.

DE 3

Sample DE 3 has a boundary between diatoms and other material, probably calcite. The area with diatoms is very homogeneous and has almost only intact diatoms. Many diatoms are completely intact and the two halves of the diatoms are still attached to each other. The diatoms are very porous. You can see some loose outer rings, but no separate inner rings.

DE 4

Sample DE 4 has more broken material than the previous samples. The 2500x picture shows that the diatoms can have many very small open pores. It also looks as if there is some carbonate In between the diatoms. This sample has also diatoms without outer ring.

DE 5

In sample DE 5 the diatoms are almost all intact. The diatoms are very ‘open’ and have big pores. The diatoms look like wheels with spokes. In the 2500x sample it can be seen that there are some needles of calcite present. In the 500x picture a different shape can be seen in the middle left side.

DE 6

Sample DE 6 has many white dots in a circular pattern on some diatoms. The material is not homogeneous and the diatoms are not always the same. It looks as if there is calcite present, for example in the 500x picture on the right upper side.

When the diatoms are seen from the side they look different than the diatoms in sample DE 2 for example.

DE 7

Sample DE 7 is inhomogeneous and consists of pennates and diatoms that look different. Especially the diatoms in the 1000x picture look different due to the white spots that are very pronounced. They are not orientated in a symmetrical way as seen in other samples.

The pennates have different orientations.

The pennates also have white specks, but they are not round and look different.

DE 8

Sample DE8 looks consistent in the type of diatoms, none have outer rings and all the inner rings have large pores. The pores are mostly not spread over the entire diatom but are grouped in a circle on the edge of the diatom. The material does not look ‘clean’. There is pennate like material present. There are not white dots as in the two previous samples.

DE 9

Sample DE 9 has many diatoms that are still intact. The halves are still attached to each other. The outer ring of the diatoms is very thin. There are almost no loose outer rings. There are not much broken fragments. The material looks very uniform and most diatoms look similar. The pennates look smaller than earlier.

DE 10

Sample DE 10 looks inhomogeneous. The diatoms are different from each other and there are lots of broken fragments. The pennates have white specks. It also looks as if there is calcite present.

DE 11

Sample DE 11 has a boundary between two layers. The outer rings are usually still attached to the diatom and often the two halves are not separated. There are also some loose rings. There are two shapes of diatoms. One with many pores and another with only a few pores which are larger. The ones with fewer pores look brighter.

Idea: The samples with a boundary (thin layers of calcite/diatoms) generally have a better preservation of the diatoms and are of higher quality?

There is a large difference between the shape of the diatoms in DE 9 and DE 11.

IBP 2

Sample IBP 2 has many diatoms in different sizes. There are many broken fragments but also intact diatoms. It could be s boundary between two layers. There are no diatoms with ‘spokes’. The diatoms look very clean. It could be that there are carbonates present between the broken fragments.

IBP 3

Sample IBP 3 is a mix of diatoms and needles of other material, probably calcite. The diatoms have no fine open structure of pores, but have larger pores that are randomly spread over the surface. It is the first sample with a clear mixture of diatoms and other material.

The pores in these diatoms might be of secondary origin.

IBP 4

Sample IBP 4 looks very inhomogeneous. In some places there are white dots on the diatoms. There is also a large piece of other material which can be seen in the first three pictures. There are diatoms with different shapes. Although the material is variable in composition the diatoms are mostly intact and not fragmented. In the 1000x and 2500x pictures it can be seen that there is some other material present between the diatoms, maybe calcite.

IBP 6

Sample IBP 6 has different sizes of diatoms. They have an open structure due to the ‘spokes’. Some white dots can be seen on the diatoms. The material does not look homogeneous. There is not much broken material. In the 2500x picture the side view of the diatom can be seen, the rings on the sides of the diatoms are not present. And are also not loose in the material. It might be a different diatom than in, for example, CO 1A.